Attenuator Module V2 for the PAM

- Frequency: 0.1-20GHz

- Attenuation: 5-BIT, max: 31 dB

- Impedance: 50Ω

Schematic

Inputs:

- RFin: Radio signal input

- +5V: positive supply voltage (+5V@4.5mA)

- -5V: negative supply voltage (-5V@5.5mA)

- CLK: Clock Signal

- DATA: serial bit-pattern for attenuator

- LE: Letch Enable

- GND

Outputs:

- RFout: Radio signal output

Components - All Case Codes are metric

2 – SMA 2Hole (142-1701-201)	[X1, X2]
5 –  Feed through cap 4-40UNC-2A (B3C153B)	[X3-7]
1 –  Turret Terminal 2-56 UNC – 2A (1595-2)	
1 –  variable attenuator (HMC1018ALP4E)	[U1]
3 – 100pF 1005 (GRM1555C2A101FA01D)	[C2,C5,C6]
3 – 1000pF 1608 (GCM1885C1H102FA16D)	[C1,C3,C4]

1 – 3.5dV equalizer 0605 (AEQ05472-T)	[EQ1]
1 –  Non-Inv. buffer VSSOP-8 (SN74LVC3G17DCUR)	[U2]
 
1 – RO4350 PCB
1 – Box 
1 – Lid
4 – Screws 3-48 UNC - 2B x 3/16 (92196A091)
4 – Screws 2-56 UNC - 2B x 1/8 (21202)
4 – Screws 2-56 UNC - 2B x 5/32 (91771A884)
1 – RF-absorber PSA 0.08’’, ca. 20 x 24 mm (MR42-0008-20)

Footprint

Ground pins surrounding the RF input and RF output are connected to the central ground pad.