Amplifier Module V3 for the PAM

- Frequency: 0.05-20GHz

- Gain: ---dB (to be measured)

- Impedance: 50Ω

Schematic

Inputs:

- RFin: Radio signal input

- +5V: supply voltage (+5V@57mA)

- GND

Outputs:

- RFout: Radio signal output

Components - All Case Codes are metric

1 – Male SMA  (AT37PET-40E0103F04)	[X2] 
1 – Female SMA (AT37PET-41E0103F04)	[X3]

1 –  Feed through cap 4-40UNC-2A (B3C153B)	[X1]
1 –  Turret Terminal 2-56 UNC – 2A (1595-2)
1 – 23 dB amp (MML098GQ4A)	[U1]
2 – 10nF 0806 (939113733510) 	[C4, C5]
1 – 100pF 1005 (GRM1555C2A101FA01D)	[C3]
1 – 1000pF 1608 (GCM1885C1H102FA16D) 	[C2]
1 – 4.7uF 3216 (TH3A475K020C5000)	[C1]
1 –  3.5dB equalizer 0605 (AEQ05472-T)	[EQ1]
 
1 – RO4350 PCB  
1 – Box 
1 – Lid
4 – Screws 3-48 UNC - 2B x 3/16 (92196A091)
4 – Screws 2-56 UNC - 2B x 1/8 (21202)
4 – Screws 2-56 UNC - 2B x 5/32 (91771A884)
1 – RF-absorber PSA 0.08’’, ca. 20 x 24 mm (MR42-0008-20)
1 – RF-absorber Urethane 0.06’’ ca. 5 x 5 mm (ARC-UD-11881-3)

Footprint of Amplifier

Ground pins surrounding the RF input and RF output are connected to the central ground pad.