Attenuator Module V1 for the PAM

- Frequency: 0.1-20GHz

- Attenuation: 5-BIT, max: 31 dB

- Impedance: 50Ω

Schematic

Inputs:

- RFin: Radio signal input

- +5V: positive supply voltage (+5V@4.5mA)

- -5V: negative supply voltage (-5V@5.5mA)

- CLK: Clock Signal

- DATA: serial bit-pattern for attenuator

- LE: Letch Enable

- GND

Outputs:

- RFout: Radio signal output

Components - All Case Codes are metric

2 – SMA 2Hole (142-1701-201)	[X1, X2]
5 –  Feed through cap 4-40UNC-2A (B3C153B)	[X3-7]
1 –  Turret Terminal 2-56 UNC – 2A (1595-2)	
1 –  variable attenuator (HMC1018ALP4E)	[U1]
3 – 100pF 1005 (GRM1555C2A101FA01D)	[C2,C5,C6]
3 – 1000pF 1608 (GCM1885C1H102FA16D)	[C1,C3,C4]
1 –  Non-Inv. buffer VSSOP-8 (SN74LVC3G17DCUR)	[U2]
 
1 – RO4350 PCB
1 – Box 
1 – Lid
4 – Screws 3-48 UNC - 2B x 3/16 (92196A091)
4 – Screws 2-56 UNC - 2B x 1/8 (21202)
4 – Screws 2-56 UNC - 2B x 5/32 (91771A884)
1 – RF-absorber PSA 0.08’’, ca. 20 x 24 mm (MR42-0008-20)

Footprint

Footprint of attenuator module (left) compared to footprint of commercially available evaluation board with the same attenuator (right).

S-parameters

S-parameter measurement with the VNA (N5230C) of attenuator module v1 (red) in comparison to attenuator evaluation board (gray).

Troubleshooting

- Attenuator replacement had no effect, ruling out a faulty component.

- Evaluation board performed well, confirming the attenuator model is not the issue.

- Switching off the power supply had no impact on the irregularity of the modules S21 trace. The S21 trace of the evaluation board looked smooth even with the power switched off.

- Amplifier and detector modules performed correctly, despite using the same CPW and SMA transitions.

- HFSS simulations indicated that trace-to-attenuator transitions should not significantly degrade performance.

- Two key differences found between the footprints of the attenuator module and evaluation board:

  - Ground plane in the module is narrowed over a longer distance.

  - Ground pads on the evaluation board are directly connected to the central ground pad, allowing RF energy to escape from the trace.