# Allen Telescope Array

Technical Details

01.04.2020						Version:1.3

## Location

| Name | Hat Creek Radio Observatory |
| --- | --- |
| Address | 42231 Bidwell RD, Hat Creek, CA 96040 |
| Altitude | 1019.222 m |
| Latitude | 40° 49' 02.75" N |
| Longitude | -121° 28' 14.65" W |

## Antennas

| Architecture | 42 dishes – 6.1 m offset Gregorian |
| --- | --- |
| Currently available for observations | 28 as of 1 July 2025 |
| Array maximum baseline | 300 m |
| Elevation range | 16 to 87 deg |
| Max elevation speed | 1 deg/sec |
| Azimuth range | -90 to 450 deg |
| Max azimuth speed | 3 deg/sec |
| Operating frequency | 1 – 11.2 GHz |
| Feed design | Log-periodic |
| Polarization | Dual linear |
| Feed operating temperature | 80 Kelvin |
| System temperature (Tsys) | 45 Kelvin @ 2 GHz; 60 Kelvin @ 8 GHz |
| HPBW | 3.5° @ 1 GHz; 0.58° @ 6 GHz; 20.9′ @ 10 GHz; |

## Time standard

| Time sync | GPS (Meridian II) |
| --- | --- |
| Station clock | Ultra-Stable OCXO |
| Available reference signals | 10MHz; 1PPS |

## Radio Frequency (RF)

| Cryogenic low noise amplifier | LNF-ABLNC1_15A; 35dB gain; 1 – 15 GHz |
| --- | --- |
| Post amplifier module (PAM) | 60dB gain; 0 - 63 dB variable attenuator; 0.5 dB step |
| PAM 1 dB gain compression | +8 dBm |
| Analog fiberoptic link converter | Photonic Systems; PSI 1601 |
| Fiber link noise figure | ≤ 45 dB |
| Fiberoptic 1 dB gain compression | +11 dBm |
| Fiberoptic connectors | FC/APC |
| Optical wavelength | 1550 nm |

## intermediate FREQUENCY (IF)

| Number of independent IF bands | 4 |
| --- | --- |
| IF bandwidth | 700 MHz |
| Number of tunable LO | 4 |
| Number of fixed LO | 1 |
| Frequency range of tunable LO | 16 – 26.7 GHz |
| Frequency of fixed LO | 16.012 GHz |
| AAF center frequency | 512 MHz |
| IF output power range | -10 dBm to -30 dBm |
| IF output connector | SMA |

## Control interface

| Telescope control software | Python 3.5 based library; ATATools.ata_control |
| --- | --- |
| GitHub location | https://github.com/SETIatHCRO/ATA-Utils |
| Software version | 1.0.3 |
| Requirements | 'ephem'; 'astropy'; 'numpy'; 'tftpy'; 'pyuvdata' |

## Network

| Internet connection | 2 Gbps full duplex |
| --- | --- |
| Site access | VPN; SSH |
| Available public IPs | 254 |
| VPN address | https://vpn.hcro.seti.org |